# Ouster + YellowScan LiDAR Data Workflow for Tree Volume Workflow

This document is intended to serve as a guide and source of notes for processing the LiDAR generated by the YellowScan drone mounted LiDAR system and the LiDAR data output by Ouster handheld LiDAR.

Key considerations:

- Ouster OS0 is NOT inherently georeferenced. It provides LiDAR and a low-grade IMU only. Without GNSS/INS, data is in a local SLAM coordinate frame (relative, not global).

- To make data processing / comparison / or combination easier between platforms, it may be beneficial to acquire external GNSS/IMU for the Ouster

  - One such example product is the APX-15 (single GNSS antenna, $6-12,000) or APX-18 (dual GNSS antennas, designed for LiDAR payloads, $12-20,000) which the drone set-up Dr. Marc has uses

  - Or the SBG Ellipse-D, $3k–6k, RTK + dual antenna heading, accuracy: 1–3 cm position

Starting with ArcPro Workflow:

Step 1: Generate LAS Files

- Has already been generated for the arboretum

Step 2: Clip Area of flight as necessary

ArcPro Workflow:

- Create project **Need to be in C drive, unable to make edits when located in E drive or in Tree Research sharepoint, need to test out in onedrive

- "Create LAS Dataset"

  - Add 25-04-23_Arboretum.las as the input from: E:\GSL Projects\Projects - In Progress\Arboretum Project\export\250423_Arboretum.las

  - Save the .lasd somewhere logical in your project folder: titled arb_create_las_dataset.lasd

  - Run

- “Extract LAS”

  - Input: arb_create_las_dataset.lasd

  - Target folder: new folder in project titled Extract LAS Folder

  - Output LAS Dataset: extracted_arb_las_dataset.lsd

  - Processing extent:

    - Draw extent

    - Draw bounds to area of interest within arb_create_las_dataset.lasd boundaries

  - Run

- “Classify LAS Ground” Tool

  - Input: extracted_arb_las_dataset.lasd

  - Ground Detection Method: Conservative Classification

  - Detection Algorithm: Latest

  - DEM Resolution: 10 meters (adjust if needed)

  - Compute Statistics: Enabled

  - Run

- View Ground Points Only

  - Select las dataset layer in contents pane: extracted_arb_las_dataset.lasd

  - Right click the layer > Properties

  - In Properties tab go to LAS Filter tab

  - Under Classification codes, select 2 Ground

    - If you do not see this, the classify las ground tool did not work

- Creating a Digital Terrain Model (DTM)

  - Use “LAS Dataset to Raster”

  - Input: extracted_arb_las_dataset.lasd filtered to the ground points via previous step

  - Output raster: DTM_extrac_lasda

  - Value Field: Elevation

  - Interpolation Type: binning, average, linear

  - Output data type: Floating Point

  - Sampling Type: Cell Size

    - Sampling Value: 1 (m)

    - Z factor: 1

- Creating a Digital Surface Model (DSM)

  - Use “LAS Dataset to Raster”

  - Input: extracted_arb_las_dataset.lasd filtered back to All points (right-click, under LAS Filters, select All Points)

  - Output raster: DSM_extrac_lasda

  - Value Field: Elevation

  - Interpolation Type: binning, average, linear

  - Output data type: Floating Point

  - Sampling Type: Cell Size

    - Sampling Value: 1 (m)

    - Z factor: 1

- Canopy Height Model

- **FOR CREATING INDIVIDUAL HEIGHT ESTIMATES OF TREES (weird)**

“Focal Statistics” tool: to help identify maximum canopy height of each tree

  - Start with 3x3 cell, repeat for 5x5 cell. 10x10 cell, 20x20 cell

- “Set Null” tool to remove values below 2m so that we are only looking at trees of significant height

- Extract Tree Points using “Raster Calculator” with conditions

  - Repeat for 3, 5, 10, 20

  - Look at the pixel mapping generated for each level

    - Ideally want one pixel (with height) per tree

    - In this case, somewhere between 5 and 10 seems to do the job

      - Thus i will generate a FS6,7,8,9

      - And a setnull5, 6, 7, 8, 9

      - And indv. pixel layers for 6, 7, 8, 9 to generate a better model

      - May need to do combinations of different levels for different areas

GEDI footprints -> get max height of raster underneath for LiDAR data -> get corresponding height of tallest tree in area with manual and phone

- GEDI footprints only within arb boundaries: select by location > made new layer from this selection

- Get max pixel value of LiDAR drone data from beneath GEDI footprints

  - “Zonal Statistics as Table”

Section A: Combining Ouster + YellowScan (With GNSS/INS)

1. Process YellowScan in POSPac (SmartBase/RTX)
 2. Process Ouster with GNSS/INS trajectory (e.g., APX-18)
 3. Export both as georeferenced LAS
 4. CloudCompare:
    - Initial alignment (same CRS)
    - ICP refinement on overlap
 5. Validate alignment (check stem offsets)
 
Key references:
 - Glennie et al. 2013 (LiDAR boresight calibration)
 - Wallace et al. 2016 (Forestry LiDAR accuracy)
 - Bienert et al. 2018 (QSM + TLS accuracy)

Section B: Ouster-Only Workflow (No GNSS)

1. Run SLAM (KissICP / LOAM / LIO-SAM)
 2. Export merged point cloud
 3. CloudCompare:
    - Noise filtering
    - Ground removal (CSF)
    - Segment trees
 4. Export trees
 5. R (TreeQSM):
    - Fit cylinders
    - Compute volume
 6. rTwig:
    - Correct overestimation
 7. Biomass:
    - Apply allometric equations
 
Notes:
 - Best for small plots
 - Drift increases with path length

Section C: R Scripts for Tree Volume (TreeQSM + rTwig)

Install packages:

install.packages(c('lidR','rTwig'))
 # TreeQSM is typically installed separately from source

Load libraries:

library(lidR)
 library(rTwig)

Load point cloud:

las <- readLAS('tree.las')
 plot(las)

Preprocess:

las <- lasfilter(las, Z > 0)
 las <- lasnoise(las)

Run TreeQSM (conceptual call):

# qsm <- treeqsm(las)
 # volume <- sum(qsm$cylinders$volume)

Apply rTwig correction:

# corrected <- correct_qsm(qsm)
 # corrected_volume <- sum(corrected$cylinders$volume)

Biomass estimation:

# biomass <- corrected_volume * wood_density
 # print(biomass)

Section D: Fully Working TreeQSM + rTwig Script

Install TreeQSM from GitHub:

install.packages('devtools')
 devtools::install_github('InverseTampere/TreeQSM')

Load libraries:

library(lidR)
 library(rTwig)
 library(TreeQSM)

Load LAS file:

las <- readLAS('tree.las')
 if (is.empty(las)) stop('LAS file is empty')

Preprocess point cloud:

las <- lasfilter(las, Z > 0)
 las <- lasnormalize(las, knnidw())

Convert to matrix for TreeQSM:

pts <- as.matrix(las@data[, c('X','Y','Z')])

Run TreeQSM:

qsm <- treeqsm(pts,
   patchdiam1 = 0.05,
   patchdiam2 = 0.02,
   lcyl = 3,
   FilRad = 3
 )

Extract volume:

volume <- sum(qsm$cylinder$Volume)
 print(volume)

Apply rTwig correction:

corrected <- correct_qsm(qsm)
 corrected_volume <- sum(corrected$cylinder$Volume)
 print(corrected_volume)

Estimate biomass:

wood_density <- 600  # kg/m3 (example)
 biomass <- corrected_volume * wood_density
 print(biomass)

Section E: Parameter Tuning for Ouster OS0 (Recommended)

OS0 typically has lower density and more noise than TLS. Use slightly larger patches and stronger filtering.

Recommended TreeQSM parameters for OS0:

qsm <- treeqsm(pts,
   patchdiam1 = 0.08,   # larger initial patch
   patchdiam2 = 0.03,   # finer secondary patch
   lcyl = 4,            # smoother cylinders
   FilRad = 4           # stronger filtering
 )

Additional preprocessing tips:

- Downsample to uniform density (voxel ~1–2 cm)
 - Remove isolated points (statistical outlier removal)
 - Ensure good stem coverage (walk around trees)

Why this matters:

- Larger patches reduce noise sensitivity
 - Higher filtering prevents overfitting branches
 - Improves volume stability for QSM

Section F: QSM Diagnostic Checklist (Quality Control)

Use this to verify your TreeQSM output is reliable:

- Stem straightness: main trunk should be continuous, not zig-zag
 - Cylinder overlap: minimal gaps or floating segments
 - Radius realism: no sudden spikes in diameter
 - Branch sanity: small branches not over-thickened
 - Volume stability: rerun with slight parameter changes → similar volume
 - Visual check: overlay cylinders on point cloud (CloudCompare)

Common failure signs:

- Inflated volume → too much noise / poor filtering
 - Broken stem → insufficient point coverage
 - Excess branches → overfitting (patch size too small)

Quick fixes:

- Increase patchdiam1/2
 - Increase FilRad
 - Improve preprocessing (denoise, downsample)